CISCO PACKET TRACER

Having successfully set up a network using a Router, Switch, and end devices, this document enlists the steps to connect two or more Routers followed by setting up a DHCP server and an HTTP server

Connecting 2 or more routers together

- Click on one of the routers and go to physical tab

- Turn the power switch off

- Install the component WIC-2T by dragging it to one of the slots

- Power on the Router and wait till it boots

- Go to config section and under the interface header to new ports would be added

- After the power on, all ports need to be reset

- To program the serial port, turn it on and set the clock rate (say 128000)

- Set the ip address to some value (say 10.0.0.1) and set the subnet as well

- Come out to the design window and similarly program all other routers

- To connect two routers pick up the serial DTE wire

- After making the connections the routers RIP table has to be filled

- Click on a router and go to config tab

- Click on RIP button

- In the adjacent panel type in all the networks that are connected to that router

- Repeat the same process for all the routers. The network is now ready to use.

Setting up a DHCP Server

- Pick up a generic server from end devices and connect it to a switch along with other end devices. Set a static ip address in desktop->ip configuration

- Click on the server and go to config tab

- Fill in server name(optional), followed by a gateway ip address

- Now click on the DHCP tab on the left panel and set up a new server pool

- Switch the service on and set a pool name

- Type the default gateway and the starting ip address

- Fill the max. no. of users(say 256).

- After filling in the details press add

- Now open any other end device connected on the same switch and switch its ip configuration from static to DHCP.

- The DHCP server will successfully allocate an ip address

Connection to an HTTP service

- In the above setup DHCP server, go to  config tab and click on HTTP in left panel

- Here all the html pages are loaded

- To add a new page click on the (+) button on the right panel.

- A new file pops up (say file1.html)

- Type in the content

- Go to any end system on the same switch and open the web browser.

- Type in the ip address of the server and press enter

- The file that was loaded on the server will appear on the screen